WIRE LISP-MACHINE WRAP TESTING MACHINE

GETTING STARTED

(LOGIN LATEE)

(ED) <FORM>

<MOUSE3> TWICE, MENU SELECTION "SELECT"

... SCREEN DIVIDED INTO 2 SECTIONS.  TOP SECTION DISPLAYS

LISP FUNCTIONS,  BOTTOM SECTION DISPLAYS RUN-TIME OUTPUT.

TO SEE OTHER LISP FUNCTIONS,  TYPE M-.

CALIBRATION OF PRONGS

(CTEST-CALIBRATE)

... MOVE PROBES TO LOCAL 1A1-10 (LEFT PROBE,  THEN RIGHT PROBE).

TO MOVE PROBES,  USE [ ] \ /  FOR DIRECTION AND

<TOP> <SHIFT> <META> <CNTRL> FOR MAGNITUDE.

AFTER PROBE IS OVER 1A10-10,  USE <MOUSE1> TO DOWN/UP PROBE.

INVOKE <MOUSE2> FOR ELECTRCAL CENTERING.  IF PROBE IS NOT

MAKING CONTACT WITH PIN,  NOTHING OCCURS,  SO MOVE PROBE

AND TRY AGAIN.

THEN,  INVOKE <MOUSE3> FOR MECHANICAL CENTERING.

END CENTERING ROUTINE WITH <RUB>.  THIS POPS THE CENTERING

ROUTINE OFF THE STACK.

... DEFAULT QUSTN 1 WITH <N>.

THE RESPONSE TO QUESTN 2  -# OF BOARDS-  SHOULD BE FOLLOWED

BY <SPACE>.  (INPUT IS AN ATOM)

... <SPACE> ALSO DEFAULTS TO YES.

... "MORE" QUESTNS SHOULD BE ANSWERED WITH <SPACE>.

... AFTER CALIBRATN,  TYPE <RUB> TO POP CALIBTATN ROUTINE

OFF STACK.

PROCESS ROUTINE

(CTEST-PROCESS-WLR "<FILENAME>" 0 <OP> )

FOR 2 BOARDS ---  <FILENAME> IS  CADRWD;ICMEM3 WLR

<OP>       IS  T    (ALREADY LOADED)

FOR 4 BOARDS ---  <FILENAME> IS  CADRWD;CADR4 WLR

<OP>       IS  NIL

... ROUTINE TAKES SOME TIME IN BEGINNG TO READ-IN DATA AND SORT.

ERROR PROCEDURES

<CALL>  ABORTS ROUTINE AND RETURNS CONTROL TO MONITOR.

IF AN ERROR OCCURS, AN    APPEARS.  RESPONSES INCLUDE:

^C  WILL CONTINUE PROCESSING IF POSSIBLE.

^Z  ABORTS ERROR MODE AND RETURNS CONTROL TO THE MONIITOR.

IF THE ERROR IS DUE TO CHAOS,  (CHAOS:RESET)  WILL RE-ENABLE

CONNECTION.  THIS MAY NOT BE NECESSARY;  A SIMPLE RE-TRY MAKE

WORK.

FILE DESCRIPTIONS

RG;CTEST PLACE      CONTAINS DATA FROM RUN; I.E., BAD WRAPS.

RG;LOSER >          CONTAINS INFO RELATIVE TO RECENT RUNS.

LMIO1;CTEST >       CONTAINS

SYSTEM FUNCTIONS

(PRINT-DISK-LABEL)  GIVES MICROCODE AND LOAD AREAS FOR BOOT.

LD2 CONTAINS WLR STUFF.

(DISK-RESTORE 2)    BOOTS WLR ENVIROMENT.

<META> <CNTRL> <CNTRL> <META> <RETURN>  WARM STARTS SYSTEM.